SALAMI SAMUEL OLUWATOMIWA

CSC/2021/176

CPE 405 Assignment

Lecture Note 1

1. Which of the following best describes virtual memory?

Ans: Use of secondary storage to extend physical memory.

2. In Paging, the fixed-size blocks in physical memory are called:

Ans: C: Frames

3. The MMU performs which of the following tasks?

Ans: B: Memory Address translation.

4. The primary purpose of address decoding in memory interfacing is to:

Ans: Ensure each memory chip responds to its unique address range.

5. A processor with 20 address lines can access a maximum memory of:

Ans: C: 1MB

6. DRAM requires a special control because it needs

Ans: B: frequent refreshing of stored data.

7. The time required for memory to output data after receiving an address is called:

Ans: B: Access time

8. A system with 15 address lines can address:

Ans: C: 32k

9. Which memory requires periodic refreshing?

Ans: D: DRAM

THEORY QUESTIONS

1.Explain the concept of memory fragmentation. Distinguish between internal

and external fragmentation.

Memory Fragmentation: Memory fragmentation occurs when free memory exists, but it is wasted or broken into pieces, making allocation inefficient.

- Internal Fragmentation: Unused space within a fixed-sized memory block assigned to a process.

- External Fragmentation: Unused space between blocks that is too small/scattered for new processes.

2. Describe the differences between paging and segmentation in memory management.

Answer:

- Paging is a memory management technique used in modern operating systems to implement virtual memory. It divides the virtual memory of a process into fixed-size blocks called pages and the physical memory into blocks of the same size called page frames.

- Segmentation: Unlike paging, segmentation divides memory into variable-sized units called segments, based on the logical divisions of a program (such as functions, objects, or libraries). While paging is invisible to the programmer, segmentation often relates to how a programmer views the structure of their code.

3. A large program cannot fit entirely in physical memory. Explain how virtual

memory allows it to run efficiently.

Answer:

Virtual memory allows systems to run programs that exceed the size of physical RAM by swapping unused memory pages to disk. Virtual memory extends the size of available memory by using disk space as an overflow area (called swap space).

4. A memory-intensive application is suffering from frequent page faults. Identify two possible solutions.

Ans:

- Adjust page Replacement Algorithm.

- Increase physical Memory.

Lecture Note 2

1. The primary purpose of address decoding in memory interfacing is to:

Ans: Ensure each memory chip responds to its unique address range.

2. A processor with 20 address lines can access a maximum memory of:

Ans: C: 1MB

3. The control signal used to enable a memory write operation is:

Ans: C: WE

4. Partial address decoding leads to which of the following problems?

Ans: B: Address aliasing

5. DRAM requires a special controller because it needs:

Ans: Frequent refreshing of stored data.

6. In a 16-bit processor interfaced with 8-bit memory chips, how many chips are needed to store one 16-bit word?

Ans: 2

7. The signal used to control the high byte during memory access in a 16-bit microprocessor is:

Ans: B: BHE

8. Which memory has the simplest interfacing requirements?

Ans: C: SRAM

9. The time required for memory to output data after receiving an address is called:

Ans: Access time

10. Memory-mapped I/O uses:

Ans: Shared memory and I/O address space - B

11. Direct Memory Access (DMA) is mainly used to:

Ans: B: Allow I/O device to access memory directly

12. Interrupt-driven I/O reduces:

Ans: B: CPU idle time

13. Which bus carries the address of the data the CPU wants to read/write?

Ans: C: Address bus

14. Which of the following describes the programmed I/O?

Ans: A: CPU waits for device readiness.

15. Which bus carries signals such as Read/Write and Interrupts?

Ans: Control bus (C)

16. Which I/O method is best for high-speed data transfer?

Ans: C: DMA

17. PCI Express (PCIe) is what type of bus protocol?

Ans: B: Serial

18. The purpose of an I/O module is to:

Ans: B: Convert device signals to CPU readable format.

19. Which protocol uses SDA and SCL lines? Ans: C: I2C

20. Bus arbitration is used to: Ans: B: Decide which device controls the bus.

21. In DMA transfers, the CPU is: Ans: C: Not Involved in data movement.

22. Which of the following I/O techniques requires the CPU to continuously check the device status? Ans: C: Programmed I/O.

23. The I/O method most suitable for high-speed devices like SSDs and graphics cards is: Ans: C: Direct Memory Access.

24. Memory-mapped I/O means: Ans: A: Devices share the same address space with main memory.

25. Which of the following BEST describes an I/O module? Ans: B: Hardware that Intermediates between the CPU/bus and peripheral.

26. PCIe communication is: Ans: B: Serial, point to point.

27. The signal used for flow control in hardware-based communication is: Ans: C: RTS / CTS

28. Which bus protocol uses lanes (x1, x4, x16)? Ans: C: PCIe

29. An interrupt that includes the address of Interrupt service routine is called: Ans: B: Vectored Interrupt.

30. SATA drives use which controller Interface? Ans: B: AHCI

31. In I2C communication, the number of wires used is: Ans: B: 2

Theory

1. Explain three differences between memory mapped I/O and Isolated (port-mapped) I/O

i) Address Space Used

- Memory-Mapped I/O: I/O devices share the same address space as main memory. Some memory addresses are reserved to represent device registers.

- Isolated (Port-Mapped) I/O: I/O devices use separate I/O address space, different from memory addresses.

ii) Instructions Used to Access Devices

- Memory-Mapped I/O: Uses normal memory instructions such as LOAD, STORE, MOV to read/write devices.

- Isolated (Port-Mapped) I/O: Uses special I/O instructions like IN and OUT (commonly in x86 systems).

iii) Hardware and Flexibility

- Memory Mapped I/O:

- Simpler hardware design.

- More flexible (can use all memory instructions, arithmetic, etc).

2. Describe Interrupt-Driven I/O and outline two advantages over Programmed I/O

Ans:

Interrupt-Driven I/O is an I/O method where the device interrupts the CPU when it is ready or finished, instead of the CPU continuously polling the device to check its status

Two advantages over programmed I/O:

- Better CPU utilization (no busy waiting).

- Faster response and higher efficiency.

3. What is Direct Memory Access (DMA)? List two devices that commonly use DMA.

Ans:

Direct Memory Access (DMA): It is a technique that allows I/O devices to transfer data directly between device and main memory without CPU involvement.

Two devices that use DMA:

- Hard disk/SSD

- Network Interface Card (NIC).

4. Three hardware components used in I/O Interfacing:

- I/O interface (Controller)

- Data register

- Status register

  - I/O Interface Controller: Connects the CPU to the I/O device and manages data transfer between them.

  - Data Register: Holds the data being transferred between the CPU and the I/O device.

  - Status Register: Stores information about the device state (ready, busy, error, etc) so the CPU knows when to read or write.

5. Differentiate between Synchronous and Asynchronous Interfacing.

Ans:

Synchronous vs Asynchronous Interfacing:

  - Synchronous Interfacing: Data transfer is controlled by a common clock signal shared between CPU and device.

  - Asynchronous Interfacing: Data transfer occurs without a common clock, using handshaking signals to coordinate.

| Feature | Synchronous | Asynchronous |
| --- | --- | --- |
| Clock | Uses shared clock | No shared clock |
| Control | Fixed timing | Handshaking Signal |
| Flexibility | Less flexible | More flexible |
| Speed | Faster | Slower |

6. Describe the components of an I/O module

Ans:

i) Data Register: Stores data being transferred between CPU & I/O device.

ii) Control Register: Holds control commands sent from CPU to the device.

iii) Status Register: Indicates device status such as ready, busy, or error.

iv) Address Decode: Selects the correct I/O device or register based on the address.

v) Interrupt Logic: Generates interrupt signals to notify the CPU when service is needed.

7. Discuss the advantages of serial bus protocols over parallel bus protocols.

Ans:

Serial bus protocols have several advantages over parallel bus protocols, especially in modern computer and embedded systems:

- Fewer wires and pins
Serial buses transmit data over one or a few wires, while parallel buses require many data lines. This reduces hardware complexity, connector size, and overall cost.

- Lower electromagnetic interference (EMI)
Fewer signal lines and controlled signaling (often differential) reduce crosstalk and noise, making serial communication more reliable.

- Higher data rates over long distances
Parallel buses suffer from signal skew (timing differences between lines) at high speeds. Serial buses avoid this issue and can operate reliably at much higher speeds and over longer cable lengths.

- Simpler PCB layout and routing
With fewer traces to route and match, serial buses make printed circuit board (PCB) design easier and more compact.

- Better scalability and flexibility
Serial protocols are easier to extend, upgrade, and adapt to new speed requirements without drastically increasing hardware complexity.

- Lower power consumption
Driving fewer signal lines generally requires less power compared to parallel buses.

- Improved reliability
Many serial protocols include built-in error detection and correction mechanisms, improving data integrity.

8.

9. A microcontroller needs to interface with four low-speed sensors that require minimal wiring          and support multiple slaves on the same bus. Which interfacing protocol is most suitable and why?

Ans: I²C

Reason: I²C is the best choice because it uses only two wires, supports multiple sensors on the same bus through addressing, and is well-suited for low-speed sensor communication.

10. An engineering student wants to connect a high-speed external display module (requiring fast data transfer) to a microcontroller. Programmed I/O results in terrible performance. Which interfacing technique should be used and why?

Ans:

Preferred technique: Direct Memory Access (DMA)

Reason: Direct Memory Access (DMA) should be used because it enables fast data transfer to the display while reducing CPU workload and improving overall performance.

11. A system using UART sometimes loses data due to overflow when transmitting large packets. Which handshaking mechanism can prevent this issue

Ans:

Hardware handshaking (RTS/CTS) can prevent this issue.

12. A student wants to connect a microcontroller to a device several meters

away. Noise is an issue. Which interfacing technique is most appropriate

Ans:

Differential serial communication

13. An old parallel-port printer using handshaking signals (ACK, BUSY, STROBE) stops working after connecting to a new PC. What is the likely cause and solution?

Ans:

The printer fails because the new PC lacks proper parallel-port handshaking support; restoring a true LPT interface or upgrading the printer fixes the issue.

14.

15. A 16-bit CPU needs to read 16-bit instructions from

Ans:

Use two EPROMs in parallel:

- One EPROM provides the lower 8 bits (D0–D7).

- The other provides the upper 8 bits (D8–D15).

Connect address lines:

- Both EPROMs share the same address lines (A0–An) from the CPU so they output the same instruction word simultaneously.

Control signals:

- Connect chip enable (CE) and output enable (OE) of both EPROMs to the CPU’s memory control signals.

- This ensures both EPROMs are read at the same time.

Data bus connection:

- The lower EPROM’s outputs → CPU D0–D7

- The upper EPROM’s outputs → CPU D8–D15

16. A system designer used a memory chip with 150 ns access time, but

the CPU requires data within 80 ns. The system fails to boot. Explain the

cause of failure and propose TWO solutions

Ans:

Cause of Failure:
The memory chip has an access time of 150 ns, meaning it takes 150 ns to provide valid data after the CPU requests it. The CPU requires data within 80 ns, so the memory is too slow. As a result, the CPU reads invalid or undefined data, causing the system to fail to boot.

Two Possible Solutions:

- Use faster memory:

  - Replace the 150 ns memory with a chip that has an access time ≤ 80 ns, so the CPU can read data on time.

- Use wait states or memory buffering:

  - Introduce wait states in the CPU’s memory cycle, allowing extra time for the slow memory to respond.

  - Alternatively, use a fast cache or buffer (like a latch) to temporarily store data for the CPU.

17. Explain bus arbitration and list two arbitration techniques

Ans:

Bus Arbitration is the process of controlling access to a shared bus in a system with multiple devices (like CPUs, memory, or I/O) to prevent conflicts when more than one device wants to use the bus at the same time. Only one device can use the bus at a time, so arbitration ensures orderly communication.

Two Common Bus Arbitration Techniques:

- Daisy Chaining (Serial Arbitration):

- Centralized Parallel Arbitration (Using a Bus Controller):

18. A high-speed SSD needs to directly transfer large data blocks to memory without CPU involvement. Which I/O method and bus protocol are most suitable?
Ans:

I/O Method: Direct Memory Access (DMA)

Bus Protocol: PCI Express (PCIe)

19. An embedded system needs to connect multiple sensors with limited wiring and low power. Which bus protocol is most appropriate and why?

Ans:

I²C is ideal for connecting multiple sensors in embedded systems.
It uses only two wires (SDA and SCL), minimizing wiring.
Supports multiple devices on the same bus via unique addresses.
Consumes low power and is easy to implement.

20. A microcontroller must communicate with a high-speed display requiring full-duplex transmission. Which protocol should be used and why?

Ans:

SPI (Serial Peripheral Interface) is the ideal protocol for connecting a microcontroller to a high-speed display because it supports full-duplex communication, allowing data to be sent and received simultaneously. It provides fast data transfer rates necessary for smooth display updates, uses simple hardware connections (MOSI, MISO, SCK, CS), and is widely supported by both microcontrollers and display modules, making it efficient and reliable for high-performance applications.

21. A network card must interrupt the CPU when a new packet arrives. Which I/O technique is used?

Ans:

The I/O technique used is Interrupt-Driven I/O.

22. A legacy system needs to connect multiple expansion cards like sound and network cards. Which bus protocol is best?

Ans:

The best bus protocol is PCI (Peripheral Component Interconnect).

23. Discuss the roles of the CPU, memory, and I/O controller in a typical I/O operation.

Ans:

In a typical I/O operation, the CPU, memory, and I/O controller each play distinct but coordinated roles:

CPU: Initiates and controls I/O operations, processes data, and handles interrupts.
Memory: Temporarily stores data being transferred between the CPU and I/O devices.
I/O Controller: Manages communication with devices, converts signals, and notifies the CPU when operations complete.

24. A company experiences slow performance because the CPU constantly waits for I/O operations to finish. What interfacing technique would you recommend and why

Ans:

Use interrupt-driven I/O to prevent the CPU from wasting time waiting, enabling efficient multitasking and faster system performance.

25. Explain the three major I/O data transfer techniques

Ans:

he three major I/O data transfer techniques are Programmed I/O, Interrupt-Driven I/O, and Direct Memory Access (DMA):

1. Programmed I/O is a method where the CPU is actively involved in the data transfer process. It continuously checks the status of the I/O device in a loop, waiting until the device is ready to send or receive data. During this time, the CPU cannot perform other tasks effectively, which makes this technique simple but inefficient, especially for slow devices or large data transfers.

2. Interrupt-driven I/O improves upon programmed I/O by allowing the CPU to perform other tasks while the I/O device is preparing data. The device generates an interrupt signal to the CPU when it is ready, prompting the CPU to temporarily halt its current task, service the I/O, and then resume its previous operation. This reduces wasted CPU cycles and allows for more efficient multitasking.

3. Direct Memory Access (DMA) takes efficiency a step further by allowing the I/O controller itself to transfer data directly between memory and the device without continuous CPU involvement. The CPU initiates the transfer and is only interrupted when the operation is complete. DMA is particularly effective for high-speed or large-volume data transfers because it frees the CPU from being involved in every step of the data movement, significantly improving overall system performance.

26. Describe four functions of an I/O module.

Ans:

1.Control and timing: Coordinates data exchange between device and system bus. 2.Device communication: Sends commands and receives status from peripherals.  3.Buffering: Absorbs speed differences between CPU and device.

4.Error detection: Uses mechanisms such as parity, CRC (cyclic redundancy check), and status flags

27. Compare PCI and PCIe bus architectures.

Ans:

| Feature | PCI | PCIe |
| --- | --- | --- |
| Transfer Mode | Parallel | Serial |
| Bus Type | Shared | Point-to-point |
| Bandwidth | Lower | Very high and scalable |
| Communication | Multi-drop | Lanes (x1, x4, x8. x16) |
| Interrupts | Line-based | Interrupts(MSI/MSI-X |

28. A company is designing a high-speed data logging system that captures sensor data at 500 MB/s. Using programmed I/O causes heavy CPU usage, and interrupt-driven I/O still creates a performance bottleneck. Which I/O method is most suitable, and why?

Ans:

The most suitable I/O method for this high-speed data logging system is Direct Memory Access (DMA).

DMA solves this problem by allowing the I/O controller to transfer data directly between the sensor and memory without involving the CPU for each byte or block. The CPU only initiates the DMA transfer and is notified when the operation is complete. This approach minimizes CPU usage, handles very high-speed data efficiently, and ensures reliable logging without data loss.

29. An embedded system uses several low-speed sensors (temperature, humidity, pressure). It requires minimal wiring and must support multiple devices on the same bus. Which protocol would you recommend between I²C and SPI? Justify

Ans:

For this embedded system, I²C (Inter-Integrated Circuit) is the recommended protocol.

I²C is ideal because it requires only two wires, SDA (data) and SCL (clock). Regardless of the number of sensors, minimizing wiring complexity. It also supports multiple devices on the same bus by assigning each sensor a unique address, allowing easy integration of several low-speed devices like temperature, humidity, and pressure sensors. In contrast, SPI requires separate chip-select lines for each device, which increases wiring and pin usage, making it less suitable for a multi-device setup with low-speed sensors.

30. A server currently uses SATA SSDs but needs a significant performance increase for database workloads. Which interface should be adopted, and why?

Ans:

The server should adopt NVMe SSDs over PCIe

Reason: Switching to NVMe over PCIe significantly improves throughput and reduces latency, delivering the high performance required for demanding database operations compared to SATA SSDs.

31. A system experiences bus congestion because multiple devices are sending frequent requests. The designer is considering switching from a shared-bus architecture to a point-to-point architecture. Which bus architecture is being considered, and how does it reduce congestion?

Ans:

The designer is considering a point-to-point bus architecture.

Reason: A point-to-point architecture connects each device directly to the CPU or memory through dedicated links rather than sharing a single bus. This allows simultaneous data transfers between different device pairs, effectively reducing congestion and improving overall throughput because devices no longer compete for a single shared bus.

32. A parallel-port printer uses READY/BUSY and ACK/NACK signals for handshaking. The printer suddenly stops responding after the system motherboard is replaced. Identify the likely cause and propose a fix.

Ans:

Likely Cause: The printer fails because the new motherboard lacks proper parallel port handshaking support

Proposed Fix: adding a compatible parallel port card or switching to a modern interface resolves the issue.

Lecture Note 3

1. RAID 0 provides:

Ans: (B) High performance only

2. Which RAID level requires a minimum of 3 disks?

Ans: (B) RAID 5

3. RAID 6 can survive how many disk failures?

Ans: (C) 2

4. Which RAID uses mirroring and striping simultaneously?

Ans: (C) RAID 10

5. The main disadvantage of RAID 5 is:

Ans: (B) Slow write performance

6. Which of the following RAID levels provides no redundancy?

Ans: (A) RAID 0

7. The process of storing parity information across multiple disks is associated with:

Ans: (C) RAID 5

8. RAID 10 requires a minimum of:

Ans: (C) 4 disks

Theory:

- Explain the concept of parity in RAID systems.

Ans:

Parity is a method used in RAID to ensure that if one hard drive fails, your data doesn’t vanish into the digital void. It is the redundancy information that can reconstruct lost data.

- Distinguish between RAID 0 and RAID 1.

Ans:

RAID 0 (Stripping) splits data evenly across two or more disks (stripes), with no parity and no redundancy.

Advantages:

• Very high read/write performance

• Efficient use of disk capacity (100%)

Disadvantages:

• No fault tolerance

• Used in High-speed systems (video editing).

RAID 1 (mirroring) duplicates data on two or more disks. Written exactly the same on two or more disks (mirrors).

Advantages:

• High data safety (fault-tolerant)

• Simple recovery—data still intact if one disk fails

• Faster read performance (can read from any mirror)

Disadvantages

• Higher cost (requires double storage)

• Write performance slightly slower • Used in Financial records, server.

|  | RAID 0 | RAID 1 |
| --- | --- | --- |
| Category | Stripping | Mirroring |
| Description | Non-redundant | Mirrored |
| Disks required | N | 2N |
| Data availability | Lower than single disk | Higher than RAID 2, 3, 4, 5, lower than RAID 6 |
| Large I/O data transfer capacity | Very high | Higher than single disk for read; similar to single for write |
| Small I/O requirement rate | Very high for both read and write | Up to twice that of single for read; similar to single disk for write |

- Describe RAID 10 architecture and its benefits.

Ans:

RAID 10 (1+0) – Mirroring + Striping: Combines RAID 1 and RAID 0. Requires minimum 4 disks.

Advantages:

• Very high read/write performance

• High fault tolerance: can survive multiple disk failures (one per mirror)

Disadvantages:

• 50% usable capacity

• Expensive (many disks)

• Used in Mission-critical enterprise system

- A company needs fast database performance and high fault tolerance which RAID level should they choose and why?

Ans: RAID 10

It combines RAID 1 and RAID 0. It has very high read/write performance and high fault tolerance.

- A storage sever with 4 disks must support two simultaneous disks failures. Which RAID level is best?

Ans:  RAID 6

It can tolerate two simultaneous disk failures and is used in enterprise servers (large capacity disks).

- A multimedia company needs extremely fast read/write speed and does not care about redundancy. Which RAID level fits?

Ans: RAID 0

It has very high read/write performance with no redundancy. It is used un high speed systems.

- A sever system must remain operational even if two of its hard disks fails. Which RAID level would you choose and why?

Ans: RAID 6

It can tolerate two simultaneous disk failures and is used in enterprise servers (large capacity disks).